SCO(r) Hi-Res Keyboard Mouse	Supplement

Release	and Installation Notes

Version	1.0.0

15 December 1990

1.	Preface

The	SCO Hi-Res Keyboard Mouse Supplement provides your Open	Desktop(r)

or UNIX(r) system with a device driver that	enables	your computer to

use	the full capabilities of the high-resolution keyboard mouse.

2.	Configuration Requirements

You	need the following resources to	install	and use	the SCO	Hi-Res

Keyboard Mouse Supplement:

15 December	1990/1.0.0	    - 1	 -  The	Santa Cruz Operation, Inc.

Release Notes

_____________________________________________

|COMPUTER      | an industry-standard 386 or |

|	       | 486 computer		     |

|______________|_____________________________|

|OPERATING     | SCO UNIX System V/386	     |

|SYSTEM	       | Release 3.2 Version 2.0,    |

|	       | Open Desktop Release 1.0.0  |

|	       | (with or without the UFE    |

|	       | Maintenance Supplement), or |

|	       | Open Desktop Release 1.1.0  |

|	       | (with or without Update F). |

|	       | For either platform, be     |

|	       | sure that you have	     |

|	       | installed the Extended	     |

|	       | Utilities Package.	     |

|______________|_____________________________|

|DISK CAPACITY | no additional storage space |

|	       | is needed for this supple-  |

|	       | ment			     |

|______________|_____________________________|

|RAM	       | no additional RAM is needed |

|	       | for this supplement	     |

|______________|_____________________________|

|MOUSE	       | a high-resolution keyboard  |

|	       | mouse			     |

|______________|_____________________________|

3.	Supported Mouse	Devices

The	SCO Hi-Res Keyboard Mouse Supplement has been tested on	the fol-

lowing mouse devices:

o Microsoft(r) Serial-PS/2 Compatible Mouse

o LogitechTM Model No. CA-93-6MD

Other high resolution mice may work	but are	not tested or supported

with this release.

4.	Installing the SCO Hi-Res Keyboard Mouse Supplement

Install the	SCO Hi-Res Keyboard Mouse Supplement after you have

installed Open Desktop or the UNIX system platform.	 If you	have

already configured a mouse on the system, first remove it with the

mkdev mouse	utility	before you install the SCO Hi-Res Keyboard Mouse

Supplement.

Follow these steps to install the SCO Hi-Res Keyboard Mouse	Supplement

using the custom(ADM) utility:

1.  Reboot the system and	enter single-user mode.

2.  Type custom and press	<Return>.  Select Install from the initial

custom menu by pressing <Return>, and	then press <Return> again

to select A New Product from the next	menu.  Press <Return> to

15 December	1990/1.0.0	    - 2	 -  The	Santa Cruz Operation, Inc.

SCO Hi-Res Keyboard Mouse	Supplement

choose the Entire Product option from	the displayed list.  You

are prompted to insert Distribution Floppy Volume 1.

3.  Insert the SCO Hi-Res	Keyboard Mouse Supplement diskette into

floppy drive 0 and press <Return> to continue.  You see the mes-

sages	Installing custom data files...	 and Executing Product

Prep Script.	You also see messages describing where custom is

saving backup	copies of files	before extracting new files.  You

are prompted to press	any key	to continue.

4.  Press	any key.  The message Creating file lists...  appears, and

you are prompted to insert the SCO Hi-Res Keyboard Mouse Supple-

ment Floppy Volume 1.

5.  Check	to make	sure that the diskette is inserted properly and

press	<Return> to continue.  You see the message Extracting

files... and,	after the volume is extracted, a restricted rights

legend appears. The messages Installing SCO Hi-Res Keyboard

Mouse	Supplement and Updating	System Configuration...	also

appear.

6.  At this point	in the installation, you are prompted if you want

to configure the system to use a high-resolution keyboard mouse.

The supplement cannot	take effect until you configure	the sys-

tem.	You can	choose to do so	now, or	you can	use the	mkdev

mouse	utility	later.	Answer y or n.

7.  If you answered n, skip to the step where you	are prompted to

relink the kernel.

8.  If you answered y, you see Mouse Initialization Program and a

numbered menu.

9.  At the prompt, enter the number next to the menu option Add a

mouse	to the system.	A list of supported mouse devices appears.

10.  Select the High Resolution Keyboard Mouse from the list.  You

are prompted to enter	all of the ttys	that you want associated

with the high-resolution keyboard mouse.

11.  Enter	multiscreen if you want	to use multiuser mode.	You are

also asked if	you want to use	the high-resolution keyboard mouse

on any other terminal.  Answer appropriately.	 Please	refer to

``Using a Mouse'' in the SCO UNIX System V/386 Operating System

Administrator's Guide	or to ``Using a	Mouse''	in the Open Desk-

top Administrator's Guide for	more information about configuring

a mouse.  The	message	Updating system	configuration... appears.

You are prompted if you want to create a new kernel to effect

the driver change.

12.  Enter	y.  This message appears:

--------------------------------------------- begin	screen display ---

15 December	1990/1.0.0	    - 3	 -  The	Santa Cruz Operation, Inc.

Release Notes

The	UNIX Operating System will now be rebuilt.

This will take a few minutes.  Please wait.

Root for this system build is /.

The	UNIX Kernel has	been rebuilt.

--------------------------------------------- end screen display -----

13.  You are prompted to decide if	you want the new kernel	to boot	by

default.  Enter y.

14.  After	the operating sytem is rebuilt,	you are	prompted to decide

if you want the kernel environment rebuilt.  Enter y.

15.  You see the Mouse Initialization Program menu.  Enter	q to leave

the program.	This prompt appears:

--------------------------------------------- begin	screen display ---

Did	you successfully relink	the kernel upon	mouse

configuration?  (y/n)

--------------------------------------------- end screen display -----

16.  If you were not successful, enter n.	This message appears:

--------------------------------------------- begin	screen display ---

If you did not successfully	relink your kernel

after making mouse configuration changes, make sure

this problem is solved.  Once this problem has been

resolved, reinstall, and relink.

--------------------------------------------- end screen display -----

You are prompted to press <Return> to	continue.  Press <Return>.

The system is	restored to its	original state,	as it was before

you attempted	to install the SCO Hi-Res Keyboard Mouse Supple-

ment.	 You are returned to the operating system prompt.

17.  If you successfully relinked the kernel, enter y and press

<Return> to continue.	The message Press any key to continue

appears.

18.  Press	any key.  You see the message Checking file permissions

and the custom menu appears.

19.  Select Quit to exit the custom menu and then choose Yes to leave

custom.

20.  Remove the distribution volume from the floppy drive.

21.  Type reboot and press	<Return>.

15 December	1990/1.0.0	    - 4	 -  The	Santa Cruz Operation, Inc.

SCO Hi-Res Keyboard Mouse	Supplement

5.	Removing the SCO Hi-Res	Keyboard Mouse Supplement

To remove the SCO Hi-Res Keyboard Mouse Supplement,	use the	following

procedure:

1.  Log onto the system as root.

2.  Enter	custom.

3.  Move the highlight or	press r	to select Remove from the initial

custom menu.

4.  Select SCO Hi-Res Keyboard Mouse Supplement in the lower right-

hand menu by moving the highlight and	pressing <Return>.

5.  Press	<Return> to choose the package labeled All, which is

highlighted in the lower window.  You	are prompted to	verify the

removal of the All package.

6.  Press	<Return> to proceed with the removal.  The message Execut-

ing SCO Hi-Res Keyboard Mouse	Supplement Removal Script appears.

If you have not removed the mouse previously using mkdev mouse,

a message appears informing you that the removal of the supple-

ment also removes the	high-resolution	keyboard mouse from the

system.

7.  Press	<Return> when prompted to do so.  You see Removing the SCO

Hi-Res Keyboard Mouse	Supplement... and this message appears:

--------------------------------------------- begin	screen display ---

The	UNIX Operating System will now be rebuilt.

This will take a few minutes.  Please wait.

Root for this system build is /.

The	UNIX kernel has	been rebuilt.

--------------------------------------------- end screen display -----

You are prompted to decide if	the new	kernel is to boot by

default.

8.  Enter	y.  After the operating	system is rebuilt, you are

prompted to decide if	you want the kernel environment	rebuilt.

9.  Enter	y, then	press any key to return	to custom.  You	see Creat-

ing file list..., and	then the initial custom	menu appears.

10.  Select Quit to leave the menu, and then choose Yes to	leave cus-

tom.

11.  Enter	the following command to shut down the system:

/etc/shutdown

15 December	1990/1.0.0	    - 5	 -  The	Santa Cruz Operation, Inc.

Release Notes

12.  Reboot the system.

6.	Changing the Mouse Resolution Setting

The	file /etc/conf/pack.d/kbmouse/space.c contains two parameters that

affect the performance of the high resolution mouse, kbm_resolution

and	kbm_poll.  You can use a text editor to	edit the

/etc/conf/pack.d/kbmouse/space.c file and change the kbm_resolution

parameter to adjust	the resolution setting.	 Refer to your platform

documentation for information on using a text editor.  The kbm_poll

parameter, however,	should only be changed automatically by	the mkdev

mouse utility under	normal circumstances.  See the section ``Solving

Slow Mouse Response'' for more information about kbm_poll.

The	kbm_resolution parameter determines how	many reports, or counts,

are	made from the mouse to the mouse driver	each time that you move

the	mouse one millimeter.  Increasing the number of	counts per millim-

eter increases the sensitivity of mouse performance.  The allowable

values for kbm_resolution are:

High-Resolution	Keyboard Mouse

15 December	1990/1.0.0	    - 6	 -  The	Santa Cruz Operation, Inc.

SCO Hi-Res Keyboard Mouse	Supplement

______________________________________

|Counts/millimeter	    Parameter	  |

|_____________________________________|

|	     1		 kbm_resolution=0 |

|	     3		 kbm_resolution=1 |

|	     6		 kbm_resolution=2 |

|	    12		 kbm_resolution=3 |

|_____________________________________|

The	kbm_resolution parameter is set	to 1 by	default	(for 3

counts/millimeter) in etc/conf/pack.d/kbmouse/space.c.

The	parameters for a keyboard mouse	do not correspond to the same

counts/millimeter as parameters for	a high-resolution keyboard mouse:

Keyboard Mouse

15 December	1990/1.0.0	    - 7	 -  The	Santa Cruz Operation, Inc.

Release Notes

______________________________________

|Counts/millimeter	    Parameter	  |

|_____________________________________|

|	     1		 kbm_resolution=0 |

|	     2		 kbm_resolution=1 |

|	     4		 kbm_resolution=2 |

|	     8		 kbm_resolution=3 |

|_____________________________________|

The	default	kbm_resolution value for a keyboard mouse is 3,	specifying

8 counts/millimeter.  The same value specifies 12 counts/millimeter

for	a high-resolution keyboard mouse, which	causes the high-resolution

keyboard mouse to behave too sensitively.  Therefore, you must use the

mkdev mouse	utility	when you add or	remove a high-resolution mouse.

mkdev mouse	changes	the default parameters in

etc/conf/pack.d/kbmouse/space.c to the appropriate values for the

mouse that you are configuring.  You must answer y when mkdev mouse

prompts you	if you want to relink the kernel if you	want to	effect the

new	parameter values.

7.	Notes

The	following sections describe issues you should be aware of concern-

ing	the SCO	Hi-Res Keyboard	Mouse Supplement.

7.1	 Reinstalling the Link Kit

If you need	to reinstall the Link Kit after	you have installed the SCO

Hi-Res Keyboard Mouse Supplement, you must reinstall the supplement

and	relink the kernel.

7.2	 Removing the SCO Hi-Res Keyboard Mouse	Supplement

You	must remove the	high-resolution	keyboard mouse from the	system

before you remove the supplement.  Note that custom	removes	the high-

resolution keyboard	mouse for you when you remove the supplement.

7.3	 Solving Slow Mouse Response

If your mouse responds too slowly after you	install	the SCO	Hi-Res

Keyboard Mouse Supplement, use a text editor to edit the file

/etc/conf/pack.d/kbmouse/space.c.  Lower the kbm_poll value	to improve

mouse response.  The minimum value is 0x5.	For example, if	the value

0xb0 produces very slow mouse response, try	0xa0, 0x90, 0x80, and so

on in turn until you have satisfactory mouse performance.

*********************************************************************

NOTE

Some kbm_poll values lower than 0xb0 may cause your	system to freeze.

If this occurs, raise the kbm_poll value.

*********************************************************************

After modifying parameters in /etc/conf/pack.d/kbmouse/space.c, you

must relink	the kernel.  To	do this, execute ./link_unix from the

/etc/conf/cf.d directory and reboot	the system.

15 December	1990/1.0.0	    - 8	 -  The	Santa Cruz Operation, Inc.

15 December	1990/1.0.0	    - 9	 -  The	Santa Cruz Operation, Inc.

Release Notes

SCO	Hi-Res Keyboard	Mouse Supplement

Release and	Installation Notes

Version 1.0.0

1. Preface	1

2. Configuration Requirements  1

3. Supported Mouse Devices	2

4. Installing the SCO Hi-Res Keyboard Mouse	Supplement  2

5. Removing	the SCO	Hi-Res Keyboard	Mouse Supplement  5

6. Changing	the Mouse Resolution Setting  6

7. Notes  8

7.1 Reinstalling	the Link Kit  8

7.2 Removing the	SCO Hi-Res Keyboard Mouse Supplement  8

7.3 Solving Slow	Mouse Response	8

15 December	1990/1.0.0	    - i	 -  The	Santa Cruz Operation, Inc.