HP(r)  DAT Supplement

Release	and Installation Notes

Version	1.0.0

30 January 1991

1.	Preface

The	HP DAT (Digital	Audio Tape) Supplement provides	your SCO(r)

UNIX(r)  System V/386 Release 3.2 Version 2.0 with additions to the

standard SCSI tape device driver.  (A device driver	is a software pro-

gram that is configured into the UNIX operating system to provide an

interface to a specific kind of hardware device.)  These additions

enable your	computer to use	the full capabilities of the Hewlett

Packard(r)	DAT device.

The	HP DAT Supplement extends DDS (Digital Data Storage) functionality

for	the operating system.  DDS is the standard data	storage	format for

4-mm DAT devices.  This extended support includes the following

features:

o The HP	DAT device is tested and integrated with the SCO SCSI tape

device	driver.

In previous releases, the HP DAT device's extended status mes-

sages were not	recognized by the operating system and were

returned instead as errors.

o This supplement supports the ``fast search'' capabilities of the

DDS format on HP DAT devices.

The DDS format	encodes	special	data separator marks (setmarks)	on

the tape in such a way	that they can be located more quickly than

by reading the	data sequentially (as most tape	devices	are

required to do).  The HP DAT device can position a tape to a set-

mark up to 200	times faster than by sequential	data access

methods.

o This supplement supports DDS tape partitions on HP DAT	devices.

The DDS format	allows you to create two partitions on a single

tape.	These partitions are logically separate	and can	be read

and written independently (but	not simultaneously).  You can

archive two separate filesystems on one tape.	You can	also use

one partition for storing a file listing of a file archive and

use another partition for storing the actual archive.	Each tape

may be	formatted with different, fixed	partition sizes.

2.	Release	Contents

This distribution includes the following:

o one distribution floppy diskette

30 January 1991		    - 1	 -  The	Santa Cruz Operation, Inc.

Release Notes

o these Release and Installation	Notes

o a new dat(HW) manual page and modified	tape(HW) and tape(C)

manual	pages for UNIX System V/386 Release 3.2	Version	2.0

3.	Configuration Requirements

You	need the following resources to	install	and run	the HP DAT Supple-

ment:

30 January 1991		    - 2	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

________________________________________

|COMPUTER	    | an industry-standard |

|		    | 386 or 486 computer  |

|________________|______________________|

|OPERATING	    | UNIX System V/386	   |

|SYSTEM	    | Release 3.2 Version  |

|		    | 2.0		   |

|________________|______________________|

|DISK CAPACITY   | no additional	   |

|		    | storage space is	   |

|		    | needed for this sup- |

|		    | plement		   |

|________________|______________________|

|RAM		    | no additional RAM	is |

|		    | needed for this sup- |

|		    | plement		   |

|________________|______________________|

|SCSI CONTROLLER | a	standard, sup-	   |

|		    | ported SCSI con-	   |

|		    | troller for UNIX	   |

|		    | System V/386 Release |

|		    | 3.2 Version 2.0	   |

|		    |			   |

|		    | SCSI cards supported |

|		    | by SCO Extended	   |

|		    | Functionality Sup-   |

|		    | plements may work	   |

|		    | but are unsupported  |

|		    | by this supplement.  |

|________________|______________________|

|DAT	DEVICE	    | an HP DAT	device,	   |

|		    | model number	   |

|		    | HP35450A or C150xA   |

|________________|______________________|

*********************************************************************

NOTE

Although UNIX System V/386 Release 3.2 Version 2.0 supports	additional

DAT	 devices,  this	supplement supplies extended functionality for the

HP DAT device only.

*********************************************************************

4.	Installing the HP DAT Supplement

After you have installed SCO UNIX System V/386 Release 3.2v2.0 and any

other updates or supplements, use the following procedure to install

the	HP DAT Supplement.

To install the HP DAT Supplement, follow these steps:

1.  Reboot the system and	enter single-user mode.	 You must be in

single-user mode to install the HP DAT Supplement.

30 January 1991		    - 3	 -  The	Santa Cruz Operation, Inc.

Release Notes

2.  Type custom and press	<Return>.  The custom main menu	appears

with the Install option highlighted.

3.  Select Install from the initial custom menu by pressing

<Return>. A new menu appears.

4.  Choose A New Product from the	next menu by pressing <Return>.	 A

new menu appears.

Press	<Return> to choose the option Entire Product from the

displayed menu.  This	selection installs the HP DAT Supplement

package.

You are prompted to insert distribution volume 1.

5.  Insert the HP	DAT Supplement diskette	into floppy drive 0 and

press	<Return> to continue.  You see the messages Installing

custom data files... and Creating file lists....  You	are

prompted to install the HP DAT Supplement floppy volume 1.

6.  Check	that the diskette is properly inserted and press <Return>

to continue.	You see	the messages Extracting	Files... and Exe-

cuting HP DAT	Supplement Init	Script.	 This message appears:

--------------------------------------------- begin	screen display ---

The	HP DAT SCSI Device is not configured into the kernel.

Do you wish	to install it now? (y/n)

--------------------------------------------- end screen display -----

7.  If you want to configure the HP DAT device now, skip to the next

step.

Answer n if you want to configure the	HP DAT device later by

using	the mkdev tape utility.

*********************************************************************

NOTE

You	cannot use the HP DAT device until it is configured into the ker-

nel.

*********************************************************************

If you answer n, you see the message:

30 January 1991		    - 4	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

--------------------------------------------- begin	screen display ---

You	may use	"mkdev tape" to	configure the HP DAT Device

at some other time.

HP DAT Supplement installation complete.

--------------------------------------------- end screen display -----

Press <Return> to go back to custom	and then press any key to

return to the main menu.  Select Quit.  Another menu appears.

Choose Yes to leave	custom and then	remove the distribution

diskette.

8.  If you answer	y to configure the HP DAT device, the mkdev tape

utility main menu appears:

--------------------------------------------- begin	screen display ---

Tape Drive Configuration Program

1.  Install a Tape Drive

2.  Remove a Tape Drive

3.  Change default Tape Drive

Select an option	or enter q to quit:

--------------------------------------------- end screen display -----

*********************************************************************

NOTE

The	system is configured by	default	with a cartridge tape drive and	a

SCSI tape drive (ID	2). If you want	to link	the HP DAT device to the

default tape device	/dev/rct0, use mkdev tape at this point	in the

installation to remove the cartridge tape driver before you	configure

the	HP DAT device.

If you want	to configure your HP DAT device	as SCSI	ID 2, use mkdev

tape to remove the SCSI tape configured at that location.

For	more information, please see Chapter 8,	``Using	Floppy Disks and

Tape Drives,'' in the SCO UNIX System V/386	Operating System

Administrator's Guide.

*********************************************************************

To install the	HP DAT device, choose the option Install a Tape

Drive.	The following menu appears:

--------------------------------------------- begin	screen display ---

30 January 1991		    - 5	 -  The	Santa Cruz Operation, Inc.

Release Notes

Tape Drive Installation Menu

1.  Install Cartridge Tape Drive

2.  Install Mini-cartridge Tape Drive

3.  Install Qic-40 Tape Drive

4.  Install SCSI	Tape Drive

5.  Install SCSI	HP DAT Drive

Select an	option or enter	q to return to the main	menu:

--------------------------------------------- end screen display -----

9.  Choose the option Install SCSI HP DAT	Drive.	This message

appears:

--------------------------------------------- begin	screen display ---

The	SCSI HP	DAT Drive must be configured before use.

Do you wish	to configure the SCSI HP DAT Drive now?	(y/n)

--------------------------------------------- end screen display -----

10.  If you want to configure the drive, skip to step 13.

If you answer	n, this	message	appears:

--------------------------------------------- begin	screen display ---

The	SCSI HP	DAT Drive has been added to the	system

configuration files.  However, /etc/conf/cf.d/mscsi

must also be updated before	using a	SCSI HP	DAT Drive.

It is advisable to reinvoke	mkdev tape accomplish this.

--------------------------------------------- end screen display -----

The mkdev tape main menu appears.

11.  Choose q to leave mkdev tape.	 You see the message HP	DAT Sup-

plement installation complete.

12.  Press	<Return> to go back to custom and then press any key to

return to the	main menu.  Select Quit.  Another menu appears.

Choose Yes to	leave custom and then remove the distribution

diskette.

30 January 1991		    - 6	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

13.  Enter	y to configure the HP DAT drive	now.  This prompt appears:

--------------------------------------------- begin	screen display ---

What is the	ID of the controller for this device?

Select 0-7,	or 'h' for help, or 'q'	to quit:

--------------------------------------------- end screen display -----

Refer	to your	hardware manual	for information	about selecting

the SCSI device location for your HP DAT device.  Locations 2-6

are recommended.

14.  Enter	the controller ID number.  This	prompt appears:

--------------------------------------------- begin	screen display ---

Which SCSI host adapter supports this device?

Select 0 or	1, or 'h' for help, or 'q' to quit:

--------------------------------------------- end screen display -----

15.  Enter	the number of the supporting host adapter.  Now	this

prompt appears:

--------------------------------------------- begin	screen display ---

What is the	LUN of this device?

Select 0-7,	or 'h' for help, or 'q'	to quit:

--------------------------------------------- end screen display -----

16.  Choose 0.  Now you see the message The SCSI configuration has

been updated.	 A list	of special device files	appears, followed

30 January 1991		    - 7	 -  The	Santa Cruz Operation, Inc.

Release Notes

by a list of file links to the default SCSI tape device loca-

tions:

--------------------------------------------- begin	screen display ---

The	following special device files have been created:

Tape Partition 1   Tape Partition 2

================   ================

/dev/urStp0.0      /dev/urStp0.1  (Unload, Rewind)

/dev/nurStp0.0     /dev/nurStp0.1 (Rewind, No Unload)

/dev/nrStp0.0      /dev/nrStp0.1  (No Rewind)

/dev/xStp0.0       /dev/xStp0.1   (Control)

For	convenience, the following links to traditional	SCSI

tape device	names have also	been created:

/dev/rStp0	  linked to /dev/nurStp0.0

/dev/rStp0.0  linked to /dev/nurStp0.0

/dev/rStp0.1  linked to /dev/nurStp0.1

/dev/nrStp0	  linked to /dev/nrStp0.0

/dev/xStp0	  linked to /dev/xStp0.0

/dev/urStp0	  linked to /dev/urStp0.0

Press <Return> to continue.

--------------------------------------------- end screen display -----

17.  Press	<Return> to continue.

If a cartridge tape drive is configured on your system, the fol-

lowing message appears:

--------------------------------------------- begin	screen display ---

The	presence of a Cartridge	Tape Drive has been detected.

the	Cartridge Tape Drive must use the default devices to

function properly.	Therefore, SCSI	devices	will be

created without the	default	device links.

--------------------------------------------- end screen display -----

The mkdev tape main menu appears.  Skip to step 23.

18.  If there is another SCSI tape	device linked to the default tape

locations, the following prompt appears:

--------------------------------------------- begin	screen display ---

30 January 1991		    - 8	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

Another SCSI Tape Drive is currently linked	to the

default device /dev/xct0.  Would you like the current

SCSI Drive linked to the default devices? (y/n)

--------------------------------------------- end screen display -----

19.  If you answer	n, the mkdev tape main menu appears. Skip to step

23.

To link the default device to	the current SCSI drive,	answer y.

The following	message	appears:

--------------------------------------------- begin	screen display ---

Default special devices have been created with the

following links:

/dev/xct0    linked to  /dev/xStp0

/dev/rct0    linked to  /dev/rStp0 and /dev/rmt/0b

/dev/rct0    linked to  /dev/nrStp0 and /dev/rmt/0bn

--------------------------------------------- end screen display -----

20.  If there is no SCSI tape device linked to the	default	tape dev-

ice locations, the SCSI tape is linked automatically to the

default tape devices.	 The following message appears:

--------------------------------------------- begin	screen display ---

Default special devices have been created with the

following links:

/dev/xct0    linked to  /dev/xStp0

/dev/rct0    linked to  /dev/rStp0 and /dev/rmt/0b

/dev/rct0    linked to  /dev/nrStp0 and /dev/rmt/0bn

--------------------------------------------- end screen display -----

The mkdev tape main menu appears.  Skip to step 23.

21.  You may also see this	message:

--------------------------------------------- begin	screen display ---

30 January 1991		    - 9	 -  The	Santa Cruz Operation, Inc.

Release Notes

Another SCSI Tape Drive is currently listed	as the default

in /etc/default/tape.  Do you wish to change this to

the	current	SCSI HP	DAT Drive?  (y/n)

--------------------------------------------- end screen display -----

22.  Answer appropriately.	The mkdev tape main menu appears:

--------------------------------------------- begin	screen display ---

Tape Drive Configuration Program

1.  Install a Tape Drive

2.  Remove a Tape Drive

3.  Change default Tape Drive

Select an option	or enter q to quit:

--------------------------------------------- end screen display -----

23.  Select q to quit.  This prompt appears:

--------------------------------------------- begin	screen display ---

You	must create a new kernel to effect the driver

change you specified.

Do you wish	to create a new	kernel now? (y/n)

--------------------------------------------- end screen display -----

24.  If you want to create	a new kernel, skip to the next step.

If you answer	n, the following message appears:

--------------------------------------------- begin	screen display ---

To create a	new kernel, execute /etc/conf/cf.d/link_unix.

Then you must reboot your system by	executing

/etc/shutdown -i0 before the changes you have specified

will be implemented

HP DAT Supplement installation complete.

Press <Return> to continue...

--------------------------------------------- end screen display -----

30 January 1991		   - 10	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

Press	<Return> to go back to custom and then press any key to

return to the	main menu.  Select Quit.  Another menu appears.

Choose Yes to	leave custom and then remove the distribution

diskette.

25.  Enter	y to relink the	kernel.	 This message appears:

--------------------------------------------- begin	screen display ---

The UNIX	Operating System will now be rebuilt.

This will take a	few minutes.  Please wait.

Root for	this system build is /.

The UNIX	kernel has been	rebuilt

Do you want	this kernel to boot by default?	(y/n)

--------------------------------------------- end screen display -----

26.  Enter	y.  You	see the	message:

--------------------------------------------- begin	screen display ---

Backing up /unix to	/unix.old

Installing new /unix

The	kernel environment includes device node	files and

/etc/inittab.  The new kernel may require changes to

/etc/inittab or device nodes.

Do you want	the kernel environment rebuilt?	(y/n)

--------------------------------------------- end screen display -----

27.  Enter	y.  You	see messages confirming	the kernel linking.  The

message The HP DAT Supplement	installation complete appears.

28.  Press	<Return> and then any key to go	back to	custom.	 You see

the message Checking file permissions...  and	then the custom

menu appears.

29.  Select Quit to leave the custom menu,	and then choose	Yes to

leave	custom.

30 January 1991		   - 11	 -  The	Santa Cruz Operation, Inc.

Release Notes

30.  Remove the HP	DAT Supplement diskette	from the floppy	drive.

31.  Enter	shutdown -g0 -y	to reboot your system and effect the con-

figuration changes that you made during the installation pro-

cedure.  The operating system	kernel is now configured to use

the HP DAT device.

5.	Removing the HP	DAT Supplement

The	HP DAT Supplement is not designed to be	removed.

6.	Notes

The	following sections describe issues you should be aware of concern-

ing	the HP DAT Supplement.

6.1	 Reinstalling the Link Kit

If you need	to reinstall the Link Kit after	you have installed the HP

DAT	Supplement, you	must reinstall the supplement and configure the	HP

DAT	device.

6.2	 Supported Media

This supplement supports the standard, 60-meter Digital Audio Tape

media.

6.3	 Problems with tar and Reaching	End of Media

If you expect to overflow the partition in which you are writing, use

the	cpio -K	command	rather than tar(C).  Refer to the section ``Speci-

fying Volume Sizes with cpio'' later in these notes	for information

about using	cpio -K.

6.4	 IMPORTANT: Data Storage Capacity

As with any	storage	media, the effective storage capacity of the HP

DAT	device may be less than	the maximum theoretical	limit of 1.3 Giga-

bytes.  Possible causes are	error correcting information and bad tape

blocks.

To ensure reliable data storage, we	strongly advise	that you modify

the	/usr/lib/sysadm/schedule file and change the size specified in the

HP DAT entry from 1300000 to 1100000.  See the section ``Using

sysadmsh for System	Backups'' later	in these notes for instructions	on

how	to modify the schedule file and	for a display of the HP	DAT device

entry.

6.5	 Differences Between HP	DAT Devices and	Standard SCSI Tape Devices

30 January 1991		   - 12	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

The	following command is not supported on the HP DAT device:

o tape reten

The	following commands are not supported on	the HP DAT device or on

standard SCSI tape devices:

o mt(C)

o tape amount

o tapecntl(C)

The	following commands are supported only on the HP	DAT device.  Do

not	use these commands on standard SCSI tape devices:

o tape rsm

o tape wsm

o tape eod

o tape partition

6.6	 Device	Minor Numbers

The	device numbers for the HP DAT drive are	located	in the /dev direc-

tory.  The minor numbers for the HP	DAT device differ from the minor

numbers for	standard SCSI tape drives.  /dev/rStp0 for SCSI	tapes does

not	have the same minor number as /dev/rStp0 for HP	DAT devices.

The	HP DAT device numbers differ because the default /dev/rStp0 tape

device for the HP DAT device is a ``no unload'' device, unlike the

standard SCSI tape.	 Also, the HP DAT device driver	selects	partitions

through another bit	in the minor number.

Some applications and/or configuration management systems may expect

the	minor numbering	scheme to be uniform across all	SCSI tape devices.

To view the	HP DAT device minor numbers, use the ls	-l command to list

device files.  For example,	enter:

ls -l /dev/nurStp0 /dev/nrStp0.1

A device file listing appears:

crw-rw-rw-   2	root   other   46,  4  /dev/nurStp0.0

crw-rw-rw-   1	root   other   46, 68  /dev/nurStp0.1

6.7	 Using the tape	Command

30 January 1991		   - 13	 -  The	Santa Cruz Operation, Inc.

Release Notes

If you are using tape(C) and a command (such as partition) fails, the

tape command does not return an error message, and it's return value

does not indicate an error.	 The device driver, however, may print an

error message to the standard output.

6.8	 Tape Distributions and	the Tape Bootstring

The	tape install utility supplied with this	supplement, mkdev tape,	is

derived from the tape installable version of our product (for compati-

bility reasons).

If you are installing this supplement on a system that was installed

with floppy	disks, you may see the following prompt	when you are ins-

talling a cartridge	tape drive:

In /etc/default/boot:

No current boot string

Enter new string, "rm"	to remove string,

or enter q to leave current string as is:

If this prompt is not documented in	your release notes, enter q to

effect no changes.	If this	prompt is documented in	your release

notes, please follow the instructions there.  This prompt allows you

to change a	configuration file so that a cartridge tape drive may be

configured from the	Boot: prompt.  This option is only available with

some SCO operating system versions.

6.9	 Installing a Western Digital SCSI Controller

If you are installing the HP DAT device on a WD7000	controller that	is

not	your primary boot device, you must modify the SCSI configuration

file /etc/conf/cf.d/mscsi before you relink	the kernel.  This file is

inappropriately modified by	the mkdev utility.  The	mkdev utility con-

figures your tape drive to use an Adaptec controller if your primary

boot device	is not a WD7000	SCSI controller.

Use	your text editor to modify /etc/conf/cf.d/mscsi	as follows.  If

your HP DAT	device is configured as	SCSI unit 1, you see a line that

looks like this:

ad		 Stp			0 1 0

Change the line to read:

wdha		 Stp			0 10

30 January 1991		   - 14	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

Relink the kernel by entering these	two commands:

cd /etc/conf/cf.d

./link_unix

This links your HP DAT device driver to the	proper location	within the

kernel, provided that you executed mkdev tape previously to	create an

entry in the /etc/conf/cf.d/mscsi file.  Reboot your system	to effect

this change.

6.10  Specifying Volume Sizes with cpio

When you invoke cpio, use the -K option to specify the media volume

size.  See the cpio(C) manual page for instructions.  Using	the -K

option prevents cpio from hanging when it reaches the End of Tape

(EOT).  If this occurs, follow the instructions in the next	section

,``Interrupting a cpio Transfer'' to reset the tape.

6.11  Interrupting a cpio Transfer

If you press <Del> to interrupt a cpio transfer (or	other types of

access to the DAT device), the interrupted process may become an unk-

illable process in the system.

Try	to reset the tape device by following this procedure:

1.  open another login session on	another	terminal or multiscreen

2.  execute the su command to become the super user

3.  issue	a tape command similar to the following	command	(this sam-

ple command is for SCSI tape device /dev/rStp0):

tape reset /dev/xStp0

If this does not release the process, try pressing the eject button	to

manually eject the tape, or	reboot the system to clear the process and

unlock the DAT device.

6.12  Formatting Partitions	on the HP DAT Device

To create two partitions on	an HP DAT tape,	use the	following command:

tape -a partition_size	partition

partition_size establishes the size	of partition 2.	 The size of par-

tition 1 defaults to the size of the remaining tape.

Using the tape -a partition	command	to format two partitions on the	HP

DAT	device may take	a long time.  The larger the partition that you

specify, the longer	time required for formatting.  Therefore, if you

are	creating two partitions	of unequal sizes, specify the smaller size

on the command line.  For example, if you want to create one partition

that is 400	Mbytes and another that	is 900 Mbytes, enter this command:

tape -a 400 partition

30 January 1991		   - 15	 -  The	Santa Cruz Operation, Inc.

Release Notes

This command requires considerably less formatting time than the

alternative, which is tape -a 900 partition.

You	cannot specify a partition size	greater	than 900 Mbytes.  If you

need a larger partition, subtract the size that you	want from 1300

Mbytes.  Specify the difference on the tape	-a command line	to estab-

lish the size of partition 2.  Partition 1 defaults	to the size that

you	want.  If you need a partition size that is 1000 Mbytes, for exam-

ple, enter this command:

tape -a 300 partition

6.13  Performing Backups

This section discusses performing user backups with	archive	utilities

and	system backups with sysadmsh.

6.13.1  User Backups

The	HP DAT device operates similarly to other SCSI tape devices.  The

tar(C) and cpio(C) commands	execute	on the HP DAT device normally.

6.13.2  Using sysadmsh for System Backups

The	backup schedule	file /usr/lib/sysadmin/schedule	contains data

necessary for sysadmsh to perform system backups. The schedule file

includes entries that specify available media types	and drive loca-

tions. sysadmsh recognizes one entry to use	as the backup device, and

the	rest are commented out by pound	signs (#).

The	HP DAT Supplement adds an entry	to /usr/lib/sysadmin/schedule.

This entry includes	the proper tape	size for the HP	DAT device:

# 1.3 Gb DAT Device

# media /dev/rct0 k 1300000 tape erase

If your HP DAT device is linked to /dev/rct0 and you are only using

one	partition, use the following procedure if you want to perform sys-

tem	backups	using sysadmsh:

1.  From the sysadmsh menu, select Backups and then choose Schedule.

You are placed in the	/usr/lib/sysadmin/schedule file	using the

vi(C)	text editor.

2.  Comment out the default media	entry by preceding the entry with

a pound sign (#).

3.  Activate the HP DAT device entry by removing the preceding pound

sign (#).

4.  Verify that all media	entries	except for the HP DAT media entry

are commented	out.

30 January 1991		   - 16	 -  The	Santa Cruz Operation, Inc.

HP DAT	Supplement

5.  Leave	vi by entering ZZ.  You	are returned to	the Backups menu.

When you next create an unscheduled	backup using sysadmsh, choose Car-

tridge Tape	from the point-and-pick	list of	available backup media to

backup the system on the HP	DAT device.  sysadmsh provides the proper

values for block and volume	sizes for the HP DAT device.  For more

information, see schedule(ADM).

To perform system backups using sysadmsh if	your HP	DAT is not config-

ured as /dev/rct0, you must	perform	one of the following operations:

o manually enter	the device location to specify the backup media

when you are using sysadmsh.  For instructions, see Chapter 2,

``Using sysadmsh'' in the SCO UNIX System V/386 Operating system

Administrator's Guide.

o edit the schedule file	and add	the appropriate	device name entry,

using existing	entries	as a template

If your HP DAT tape	is partitioned,	you also must provide sysadmsh

with the appropriate tape sizes for	each partition.	 Currently,

sysadmsh supports one backup device.  If your tape is partitioned,

each partition on the tape appears as a separate device.  You can con-

figure only	one partition at a time.

Refer to Chapter 17, ``Backing Up Filesystems,'' in	the SCO	UNIX Sys-

tem	V/386 Operating	System Administrator's Guide for more information.

7.	Programmer's Notes

This section discusses problems accessing the HP DAT device	from a C

program using the UNIX system call interface.  These problems do not

affect user-level commands such as tar(C), cpio(C),	or tape(C).

The	following sections address problems using multiple read( ),

write( ), and ioctl( ) system calls	between	a single open( ) and

close( ) system call sequence.

7.1	 Using ioctl( )	with HP	DAT Devices

Although the problems described in this section effect HP DAT devices,

they are generic to	all SCSI tape devices.

7.1.1  Unsupported Parameters

The	ioctl( ) parameters MT_DSTATUS,	MT_AMOUNT are not functional.

7.1.2  Buffer Flushing

The	SCSI tape driver internally buffers data to enhance the	driver's

ability to keep the	tape drive streaming.  Internal	buffering means

that every time you	write to the tape, the data is not necessarily

written to the tape	physically but may instead be stored in	an

internal memory buffer.

30 January 1991		   - 17	 -  The	Santa Cruz Operation, Inc.

Release Notes

This buffer	is only	flushed	(writing physically all	data in	memory to

the	tape) upon a close( ) or fclose( ) system call.	 Consequently, the

buffer is not emptied when an ioctl( ) command (such as MT_WFM) is

issued using ioctl().  Because the buffer is not flushed, data write

requests and ioctl(	) commands are sent to the tape	drive in a dif-

ferent order than they were	received by the	kernel.	The ioctl( )

request is sent to the tape, but the previously-written data remains

in the internal buffer and is written to the tape only after the dev-

ice	is closed.

In the worst case, the device driver can overwrite data if you use the

MT_REWIND ioctl( ) command and the buffer contains data.  In this

case, after	the MT_REWIND command, the data	remains	in the buffer and

is written to the tape whenever the	device is closed.

To avoid this problem, you must close and then reopen the device

before you issue an	ioctl( ) command.  This	sequence flushes the

buffer before you issue an ioctl( )	command.  Note that you	must close

and	open the device	before a read( ) as well as a write( ) command.

This sequence is similar to	what occurs at the user-level; the device

opens as the program executes and then closes as the program ter-

minates.  Sequential data read( ) and write( ) operate normally.  The

only user-level program that issues	an ioctl( ) to the SCSI	tape dev-

ice	is tape(C).  tape(C) opens the device before using an ioctl( )

command, ensuring that the device was previously closed and	that the

buffer is empty.  Therefore, no buffer flushing problem exists at the

user-level.

30 January 1991		   - 18	 -  The	Santa Cruz Operation, Inc.

HP DAT Supplement

Release and	Installation Notes

Version 1.0.0

1. Preface	1

2. Release Contents	 1

3. Configuration Requirements  2

4. Installing the HP DAT Supplement	 3

5. Removing	the HP DAT Supplement  12

6. Notes  12

6.1 Reinstalling	the Link Kit  12

6.2 Supported Media  12

6.3 Problems with tar and Reaching End of Media	12

6.4 IMPORTANT: Data Storage Capacity  12

6.5 Differences Between HP DAT Devices and Standard SCSI	Tape

Devices  12

6.6 Device Minor	Numbers	 13

6.7 Using the tape Command  13

6.8 Tape	Distributions and the Tape Bootstring  14

6.9 Installing a	Western	Digital	SCSI Controller	 14

6.10 Specifying Volume Sizes with cpio  15

6.11 Interrupting a cpio	Transfer  15

6.12 Formatting Partitions on the HP DAT	Device	15

6.13 Performing Backups	16

6.13.1 User	Backups	 16

6.13.2 Using sysadmsh for System Backups  16

7. Programmer's Notes  17

7.1 Using ioctl(	) with HP DAT Devices  17

7.1.1 Unsupported Parameters  17

7.1.2 Buffer Flushing  17

30 January 1991		    - i	 -  The	Santa Cruz Operation, Inc.

####